FAST NUCES LAHORE

HAFIZ AHMAD

19l-1316

EMS

LAB REPORT 3

EXPERIMENT 4

MODELING OF TRANSFORMER

(A) OPEN CIRCUIT TEST

(B) SHORT CIRCUIT TEST

(C) VOLTAGE REGULATION

(D) EFFICIENCY OF TRANSFORMER

EQUIPMENT:

- 1 Phase Transformer

- Single Phase Variable AC Supply

- Power Meter

- AC Voltmeter and Ammeter

- Load Box

- Connecting Wires

INTRODUCTION:

The three quantities must be determined in order to describe the transformer model:

- The equivalent resistance of Primary and Secondary side.

- The equivalent reactance of Primary and Secondary side.

- The resistance of the core 𝑅𝑐.

- The reactance of the core (Magnetization Reactance) 𝑋m.

The mentioned quantities can be determined by performing the following two tests:

1. Open-circuit test

2. Short circuit test.

Open Circuit Test:

The resistance and inductances in a transformer are necessary to find. It is really important that one would find out these values for modeling the approximate model of transformer. In order to perform the open circuit test of the transformer, the secondary side of the transformer is left open. The full rated line voltage is being applied at the primary side of transformer. If you look at the figure drawn above it is clear that in such a test, the input current flows through the excitation branch of the transformer. The equivalent resistance of the primary and secondary windings is too small as compared to the core (𝑅𝑐 and 𝑋𝑚) so they will not create significance difference. So, all the input voltage will drop at the excitation branch. The circuit diagram for open circuit test is shown in Figure 4.3

Short Circuit Test:

The short circuit test is performed by taking care of the applied voltage. In a short circuit test the secondary side is short circuited while the input voltage is applied to the primary side of the transformer. While applying input voltage, one must take precautions not to exceed it beyond a safe level. So, it is important to make it sure that the voltage applied should be within the limits otherwise the windings of the transformer can burn out. In the short circuit test the negligible current flows through the excitation branch so most of the current flow through primary and secondary windings. The short circuit test is used to find the equivalent resistance and reactance of the primary and secondary windings. The circuit diagram for short circuit test is shown in Figure 4.4

Voltage regulation:

An ideal transformer has no series impedance in it. But in case of real transformer there is the series impedance within it. The output voltage of a transformer varies with the load even if the input voltage remains constant. So, to conveniently compare transformers in this respect, it is customary to define a quantity called as voltage regulation. Full load voltage regulation is the quantity that compares the output voltage of a transformer at no load with the output voltage at full load.

For an ideal transformer VR = 0%. In case of real transformer, the voltage regulation is not equal to zero. It varies depending on the kind of load. The circuit diagram for finding Voltage Regulation is shown in Figure 4.5

Efficiency

Transformers are also compared and judged depending on their efficiencies. The efficiency of a device is defined by the following formula.

OBJECTIVE:

- Modeling of a transformer.

- To find the core resistance and reactance by using open circuit test. (No Load Test).

- To perform short circuit test on transformer.

- To study the voltage regulation of the transformer.

- To study the efficiency of transformer.

- The Open circuit test and Short Circuit Test is carried out to determine the various losses, efficiency and also impedance, and resistance.

APPLICATION:

- By using open circuit test we can measure the loss of no-load losses means that iron losses.

- By using short circuit test we can measure the loss of full-load losses means that Cu-losses.

- We can find the efficiency by using of iron losses and cu-losses.

- Voltage regulators are used in mostly all applications as we don’t trust our ac circuits. If the input increases the output also increases so to avoid this we use voltage regulators.

PROCEDURE:

Open Circuit Test:

- Make the connection as show in diagram carefully.

- Apply the full line voltage to the primary side of the transformer.

- Measure the input voltage, input current and the power of the transformer.

- Determine the power factor of input current by using the following formula.

- 𝐂osΘ = 𝐏𝐨c/ 𝐕oc ∗ 𝐈𝐨c

- Measure the magnitude and angle of excitation impedance.

- 𝐘 = 𝟏 / 𝐑C – 𝐣 𝟏 / 𝐗𝐦

- The admittance is calculated by using 𝑉𝑜c and 𝐼oc.

𝐘 = 𝐈oc / Voc

Short Circuit Test:

1. Apply the line voltage to the primary side of the transformer and increase the voltage until the current through the secondary windings is equal to its rated value.

2. Measure the input voltage, input current and the power of the transformer.

3. Determine the power factor of input current by using the following formula.

𝐂os𝛉 = 𝐏sc / 𝐕sc ∗ 𝐈sc

4. Measure the magnitude and angle of excitation impedance.

𝐙se = 𝐑eq + 𝐣Xeq

5. The short circuit impedance is calculated by using the measured values of 𝑉sc and 𝐼sc.

𝐙𝐬e = 𝐕sc  / 𝐈sc

Voltage Regulation:

- Using your Transformer Trainer connect the circuit as shown in Figure 4.5. Turn on the power supply and adjust for exactly rated Vac as indicated by volt-meter V1

- Apply the rated Voltage on the low voltage side of transformer.

- Place all of the Resistance Module switches in their open position for zero load current.

- Measure and record the output voltage E2

- Now switch the resistance modules in such a way that the maximum load is being applied at the transformer. which is the voltage at no load

- Note down the value of E2

- Calculate the transformer regulation using the no load and full load output voltages. at this load. This is equal to full load voltage.

Efficiency:

1. Using your Transformer Trainer connect the circuit as shown in Figure 4.5.

2. Connect an ammeter on the high voltage side to measure the Current Is.

3. Switch all of the Resistance Module switches to give a constant load.

4. Turn on the power supply and adjust for exactly rated Vac as indicated by volt-meter V

5. Apply the rated Voltage on the low voltage side of transformer. 1

6. Measure and record the output voltage E2= Vs which is the current at a specified load.

7. Measure and record the output current I which is the voltage at a specified load.

8. Power factor is dependent on the type of load.

9. Calculate the specified copper losses and core losses.

10. Calculate the transformer efficiency by using calculated values

CALCULATIONS AND MEASUREMENTS:

ISSUE:

Faced issue in calculation copper and iron losses.

CONCLUSION:

- Hence, we can conclude that open circuit test on transformer gives core losses of transformer and shunt parameters of the equivalent circuit. A voltmeter (V), wattmeter (W), and an ammeter (A) are connected in HV side of the transformer.

- Short circuit test is used for finding out Core losses and open circuit test is used for finding out reactance Xm and Resistance Rc.

- Both, short circuit test and open circuit test should be performed in order to get the accurate equivalent circuit for the transformer.

POST LAB QUESTIONS

- Why the voltage on primary side of transformer is kept low?

Since the impedance of the series winding of the transformer is very small compared to that of the excitation branch, all of the input voltage is dropped across the excitation branch. ... Since the secondary of the transformer is open, the primary draws only no-load current, which will have some copper loss.

- While performing experiment on which side you referred your transformer? Primary or secondary.

While performing experiment I referred the primary side of the transformer.

3. What is voltage regulation? Why voltage regulation is important?

In electrical engineering, particularly power engineering, voltage regulation is a measure of change in the voltage magnitude between the sending and receiving end of a component, such as a transmission or distribution line.

IMPORTANCE OF VOLTAGE REGULATION

The voltage regulator is needed to keep voltages within the prescribed range that can be tolerated by the electrical equipment using that voltage. Voltage regulators also are used in electronic equipment in which excessive variations in voltage would be detrimental.

- What are the advantages offered by a three-terminal regulator?

Three terminal voltage regulators have three terminals namely input which is unregulated (Vin), regulated output (Vo) and common or a ground terminal. These regulators do not require any feedback connections.

ADVANTAGES

- The three terminal voltage regulator is conveniently used for local regulation.

- The three terminal  voltage regulator is easy to use.

- It is most efficient and reliable.

- The three terminal voltage regulator is versatile.

- It is very cheap due to mass production and easily available.

- It is compact in size, rugged and light in weight.

- The power supply design becomes easy and quick.

- What are the factors that affect the secondary voltage of a transformer?

The factors that affects the voltage are:

- The turns per volt ratio,

- The flux density,

- Frequency,

- Ampere turn and

- The coupling coefficient

6. At ideal cases what will be the percent of regulation?

At ideal cases the percent of voltage regulation is 0% (zero percent).

7. What is difference between input (line) regulation and load regulation?

| LINE REGULATION | LOAD REGULATION |
| --- | --- |
| The ability of a power-supply voltage regulator to maintain its output voltage despite variations in its input voltage | Load regulation refers to the ability to maintain a constant voltage output from a power supply despite changes or variability in the input load. Typically, we express load regulation as a percentage of the max load condition, indicating how much output will vary. |

8. Does the primary winding VA equal the secondary winding VA for every value load?

NO, it is not true that the primary winding VA equal the secondary winding VA for every value load because every load have its own value of impedances though which changes secondary voltage.